| EECS 322 | # Test 5 sample | ## Monday May 1, 2000 |
| --- | --- | --- |

| Name: |  |  |
| --- | --- | --- |

Problem 1 (20%). Given the following direct-mapped cache architecture:

All instructions use byte addresses. The address bus is 8 bits. A word size is 16 bits.

Total data cache size 4 words.

1a. (2%) How many bits is the index?

1b. (2%) How many bits is the byte offset?

1c. (2%) How many bits is the tag size?

1d. (14%) For the following instruction sequence, fill in the access bits to the data cache

| tag | index | byte offset | instruction |
| --- | --- | --- | --- |
|  |  |  | lw $1, 32($0) |
|  |  |  | lw $2, 34($0) |
|  |  |  | add $3, $1, $2 |
|  |  |  | sw $3, 44($0) |
|  |  |  | lw $4, 12($0) |
|  |  |  | lbu $5, 41($0) |
|  |  |  | beq $1, $2, 34($0) |

Problem 2 (10%). Given the following 2-way set associative cache architecture:

All instructions use byte addresses. The address bus is 8 bits. A word size is 16 bits.

Total data cache size 4 words.

2a. (2%) How many bits is the index?

2b. (2%) How many bits is the byte offset?

2c. (2%) How many bits is the tag size?

2d. (4%) For the following instruction sequence, fill in the access bits to the data cache

| tag | index | byte offset | instruction |
| --- | --- | --- | --- |
|  |  |  | lw $1, 32($0) |
|  |  |  | lbu $2, 33($0) |

Problem 3. (15%) For the following instruction sequence fill in the direct-mapped cache

The word size is 16 bits.

Memory[0]=0x1066; Memory[2]=0x1453; Memory[16]=$3=0x1776; Memory[20]=0x1914;

3a. (5%) Fill in the miss cache column.

| tag | index | byte offset | instruction | Valid or Tag Cache Miss? |
| --- | --- | --- | --- | --- |
| 000 | 00 | 0 | lw $1, 0($0) |  |
| 000 | 01 | 0 | lw $2, 2($0) |  |
| 010 | 00 | 0 | sw $3, 16($0) |  |
| 010 | 10 | 1 | lbu $5, 21($0) |  |
| 010 | 10 | 0 | lw $6, 20($0) |  |

3b. (10%) The final state of the direct mapped data cache is:

| index | valid | tag | data |
| --- | --- | --- | --- |

Problem 4. (15%) For the following instruction sequence fill in the 2-way set associative LRU cache

The word size is 16 bits.

Memory[0]=0x1066; Memory[2]=0x1453; Memory[16]=$3=0x1776; Memory[20]=0x1914;

4a. (5%) Fill in the miss cache column.

| tag | index | byte offset | instruction | Valid or tag Cache Miss? |
| --- | --- | --- | --- | --- |
| 000 | 0 | 0 | lw $1, 0($0) |  |
| 100 | 0 | 0 | sw $3, 16($0) |  |
| 000 | 0 | 1 | lbu $5, 1($0) |  |
| 101 | 0 | 0 | lw $6, 20($0) |  |
| 000 | 1 | 0 | lw $1, 2($0) |  |

4b. (10%) The final state of the 2-way set associative LRU data cache is:

| index | valid | tag | data |
| --- | --- | --- | --- |
| 0 |  |  |  |
| 1 |  |  |  |

Problem 5. (10%) Given a 1-word cache entry block size and one word wide memory bus organization (figure 7.13a, page 561), and the following access times:

1 clock cycle to send the address,

8 clock cycles to read access DRAM, 16 clock cycles to write to DRAM

1 clock cycle to to send a word

5a. (5%) What is the miss penalty for a write-through direct mapped cache?

5b. (5%) What is the miss penalty for a write-back direct mapped cache?

Problem 6 (20%). Given the following virtual memory architecture:

All instructions use byte addresses. The virtual address bus is 16 bits. A word is 16 bits.

Total page size 8 bytes. The real memory address bus is 16 bits.

6a. (2%) How many bits is the page offset?

6b. (2%) How many bits is the physical page number size?

6c. (2%) How many page table entries?

6d. (2%) How large is the page table?

6e. (12%) For the following instruction sequence, fill in the data access bits to the page table

| virtual page number | page offset | instruction |
| --- | --- | --- |
|  |  | lw $1, 32($0) |
|  |  | lw $2, 34($0) |
|  |  | lbu $5, 41($0) |

Problem 7. (10%) Assume 2K real memory, LRU, a page size of 1K and no pages loaded.

Fill in the page fault columns

| instruction | Page fault? | Flush which page? | Write flushed page to disk? | Load what new page |
| --- | --- | --- | --- | --- |
| lw $1, 32($0) |  |  |  |  |
| lw $2, 34($0) |  |  |  |  |
| lbu $5, 41($0) |  |  |  |  |
| sw $6, 1100($0) |  |  |  |  |
| lw $7, 2000($0) |  |  |  |  |
| lw $1, 4100($0) |  |  |  |  |
| lw $2, 32($0) |  |  |  |  |