| EECS 322 | # Test 5 | ## Monday May 1, 2000 |
| --- | --- | --- |

| Name: |  |  |
| --- | --- | --- |

Problem 1 (15%). Given the following “spatial” direct-mapped cache with 2 word blocks:

All instructions use byte addresses. The address bus is 10 bits. A word size is 16 bits.

Each cache entry (i.e. block size) contains 2 words. Total data cache size 16 words.

1a. (5%) How many bits is the index?

= log2 cache size in blocks = log2 8 = 3

1b. (2%) How many bits is the byte offset?

= log2 number of bytes per block = log2 word size/8 = log2 16/8 = log2 2 = 1

1c. (1%) How many bits is the tag size?

= address size – index size – byte offset size = 10 – 3 – 1 = 6

1d. (7%) For the following instruction sequence, fill in the access bits to the data cache

| tag | index | byte offset | instruction |
| --- | --- | --- | --- |
| 000 001 | 000 | 0 | lw $1, 16($0) |
| 000 001 | 110 | 0 | sw $3, 24($0) |
|  |  |  | lui $1, 12 |
| 000 000 | 111 | 1 | lbu $5, 15($0) |
|  |  |  | sltiu $1, $2, 32 |

Problem 2 (15%). Given the following 4-way set associative cache architecture:

All instructions use byte addresses. The address bus is 10 bits. A word size is 32 bits.

Total data cache size 16 words.

2a. (5%) How many bits is the index?

= log2 (cache size in blocks / N-way) = log2 (16/4) = log2 4= 2

2b. (2%) How many bits is the byte offset?

= log2 number of bytes per word = log2 word size/8 = log2 32/8 = log2 4 = 2

2c. (1%) How many bits is the tag size?

= address size – index size – byte offset size = 10 – 2 – 2 = 6

2d. (2%) For the following instruction sequence, fill in the access bits to the data cache

| tag | index | byte offset | instruction |
| --- | --- | --- | --- |
| 000 000 | 11 | 00 | lw $1, 12($0) |
| 000 000 | 11 | 01 | lbu $2, 13($0) |

2f. (5%) Increase the cache block size from 1 word to 2. How many bits is in the index?

= log2 (cache size in blocks / N-way) = log2 (8/4) = log2 2= 1

Problem 3. (15%) For the following instruction sequence fill in the direct-mapped cache

The word size is 16 bits.

Memory[0]=0x1066; Memory[8]=0x1453; Memory[16]=$3=0x1776; Memory[24]=0x1914;

3a. (5%) Fill in the miss cache column.

| tag | index | byte offset | instruction | Valid or Tag Cache Miss? |
| --- | --- | --- | --- | --- |
| 000 | 00 | 0 | lw $1, 0($0) | yes - valid |
| 011 | 00 | 0 | lw $2, 24($0) | yes – tag |
| 010 | 00 | 0 | sw $3, 16($0) |  |
| 010 | 00 | 1 | lbu $5, 17($0) |  |
| 001 | 00 | 0 | lw $6, 8($0) | yes - tag |

3b. (10%) Show all states and underline the final state of the direct mapped data cache:

| index | valid | tag | data |
| --- | --- | --- | --- |
| 00 | NY | 000011010001 | 0x10660x19140x17760x1453 |
| 01 | N |  |  |
| 10 | N |  |  |
| 11 | N |  |  |

Problem 4. (15%) For the following instruction sequence fill in the 2-way set associative LRU cache

The word size is 16 bits.

Memory[0]=0x1066; Memory[8]=0x1453; Memory[16]=$3=0x1776; Memory[24]=0x1914;

4a. (5%) Fill in the miss cache column.

| tag | index | byte offset | instruction | Valid or tag Cache Miss? |
| --- | --- | --- | --- | --- |
| 0000 | 0 | 0 | lw $1, 0($0) | yes |
| 0110 | 0 | 0 | lw $2, 24($0) | yes |
| 0100 | 0 | 0 | sw $3, 16($0) | no – flush oldest reference |
| 0100 | 0 | 1 | lbu $5, 17($0) |  |
| 0010 | 0 | 0 | lw $6, 8($0) | yes– flush oldest reference |

4b. (10%) The final state of the 2-way set associative LRU data cache is:

| index | valid | tag | data |
| --- | --- | --- | --- |
| 0 | NY | 00000100 | 0x10660x1776 |
|  | NY | 01100010 | 0x19140x1453 |
| 1 | N |  |  |
|  | N |  |  |

Problem 5. (10%) Given a two-word cache entry block size and one-word wide memory bus organization (figure 7.13a, page 561), and the following access times:

2 clock cycle to send the address,

8 clock cycles to read access DRAM, 16 clock cycles to write to DRAM

3 clock cycle to to send a word

5a. (5%) What is the miss penalty for a write-through direct mapped cache?

The miss penalty only include data reads not writes, since write-through is handled on the store instruction.

Miss penalty = 2(2 send address) + 2(8 clocks to read) + 2(3 send word) = 26 clocks

5b. (5%) What is the miss penalty for a write-back direct mapped cache?

This miss penalty includes only data reads BUT on write back the cache entry may contain a block not previously written out before a new entry can be read in (page 554).

Miss penalty = 2(2 send address) + 2(3 send word ) + 2(16 clocks to write)

+ 2(2 send address) + 2(8 clocks to read) + 2(3 clock to send word) = 68 clocks

Problem 6 (20%). Given the following virtual memory architecture:

All instructions use byte addresses. The virtual address bus is 20 bits. A word is 16 bits.

Total page size 16 bytes. The real memory address bus is 16 bits.

6a. (4%) How many bits is the page offset?

= log2 number of bytes per page = log2 16 = log2  24 = 4

6b. (4%) How many bits is the physical page number size?

= virtual address bit size – page offset bit size = 20 – 4 = 16

6c. (4%) How many page table entries?

= virtual address size/page size = 220 / 24 = 216

6d. (4%) How large is the page table?

= #entries  (bytes per entry to hold the real memory address)

= 216  21  = 217 bytes = 131,072 bytes

6e. (4%) For the following instruction sequence, fill in the data access bits to the page table

| virtual page number | page offset | instruction |
| --- | --- | --- |
| 0000 0000 0000 0000 | 1000 | sw $1, 16($0) |
| 0000 0000 0000 0001 | 0000 | lw $2, 32($0) |
| 0000 0000 0000 0001 | 0001 | lbu $5, 33($0) |

Problem 7. (10%) Assume 2048 bytes of real memory, LRU, a page size of 1024 bytes and no pages loaded in memory. Fill in the page fault columns. (Blank space implies No)

| instruction | Page fault? | Flush which page? | Write flushed page to disk? | Load what new page |
| --- | --- | --- | --- | --- |
| lw $1, 0($0) | yes |  |  | 0..1023 |
| lw $2, 1024($0) | yes |  |  | 1024..2047 |
| lbu $5, 0($0) |  |  |  |  |
| sw $6, 2048($0) | yes | 1024..2047 |  | 2048..3071 |
| lw $7, 1024($0) | yes | 0..1023 |  | 1024..2047 |
| lw $1, 0($0) | yes | 2048..3071 | Yes | 0..1023 |
| lw $2, 1032($0) |  |  |  |  |