| EECS 314 | # Extra Credit Test 1 | ## Due: Wednesday February 6, 2002 |
| --- | --- | --- |

| Name: |  | ## Email: ______________________________ |
| --- | --- | --- |

Problem 1 (18%). Assemble the following machine instructions into binary, use spaces to separate fields, and registers in their symbolic form ($ra NOT $31). Assume absolute jump addresses.

| Field 1 | Fields 2 and etc | instruction |
| --- | --- | --- |
|  |  | jal 0x400 |
|  |  | li $10, 0xdefa |
|  |  | lhu $at,26($sp) |
|  |  | b 0x400 |
|  |  | not $a2 |
| 0x12740002 |  |  |

Problem 2 (7%).

Assume each part is independent. Assume absolute jump & branch addresses (no pc relative).

Fill in only registers that changed!

What is the value of the register or memory contents after the execution of the instruction.

Assume  pc = 2020;   $s3=12;  $s4=6;  $ra=250; memory[8]=0xfedcba98; memory[12]=0x76543210;

| instruction | pc | $ra | $s3 | $s4 | memory[8] | memory[12] |
| --- | --- | --- | --- | --- | --- | --- |
| jal 400 |  |  |  |  |  |  |
| sll $s4, $s3, 2 |  |  |  |  |  |  |
| sh $s4, 8($s4) |  |  |  |  |  |  |
| move $ra, $s4 |  |  |  |  |  |  |
| slt $s3, $s4, $ra |  |  |  |  |  |  |
| bgt $s3,$s4,40 |  |  |  |  |  |  |
| ori $s3,$s4,0x0001 |  |  |  |  |  |  |

Problem 3. (25%) Translate the following C code into MIPS. Please comment your code. Assume signed unless defined otherwise. x is $s0; y is $s1; s is $s2; t is $s3; r is $s4; p is $s5; d is $s6.

No pseudo-assembler instructions allowed.   Points will be taken off for assembler syntax errors.

register unsigned int x, y; register int s, **t; struct { int a[3]; short b; char c; } r, *p; char d;

- x  += ( x  –  y ) + (s – 5);

- **t = d;

- x  =  (x  >=  3)?  y*3 : 0x40 + 3;

- for(x=y;   x < y+2;   x+=8 )   {  y *= 3; }

(d)   r.c = r.b + r.a[x];

(e)  p–>c = p–>b + p–>a[x];

Problem 4. (25%)  Translate the following code and add comments

No pseudo-assembler instructions allowed.   Points will be taken off for assembler syntax errors.

char	*strcpy(char *s, char *t)  {

register int  i;

i=0;

while((s[i] = t[i]) != ‘\0’) i++; /* note: “=” NOT “= =” */

return s;

}

- Write the prolog

- Write the body

(c) Write the epilog

Problem 5. (10%)  Translate the following global variables and assign the location counter beginning at 4000

- short x=0x1914; short *s=&x; short **p = &s;

| Location counter | Assembler definitions |
| --- | --- |
| 4000 |  |

- struct keyword {

char		**argv;

int		(*daytab)[13];

int		*(montab[13]);

void		(*strcpy)();

struct	keyword *next;

}   fp;

| Location counter | Assembler definitions |
| --- | --- |
| 4000 |  |

Problem 6. (15%)  Given the following instruction sequence in the table below.

Assume the (alu and slt instructions are 5 clocks); (loads 10 clocks); (stores 20 clocks); (jumps 2 clocks); (branches 4 fall through/8 for branch);

(a)  How many different timing paths? _________

(b)  Show the best case timing path through the code showing annotations and total.

(c)  Show the worst case timing path through the code showing annotations and total.

(d)  What values will make this code execute the worst case?

| instruction | best case | worst case |
| --- | --- | --- |
| slti $t0,$a0,3 |  |  |
| slt $t1,$a0,$t0 |  |  |
| beq $t1,$0,L1 |  |  |
| addi $t2,$zero,5 |  |  |
| L1: beq $a2,$t1,L2 |  |  |
| addi $a1,$a1,3 |  |  |
| L2: addi $s1,$zero,10 |  |  |
| Total Time |  |  |