| EECS 314 | # Test 1 | ## Due: Wednesday February 6, 2002 |
| --- | --- | --- |

| Name: |  | ## Email: _____________________________ |
| --- | --- | --- |

Problem 1 (18%). Assemble the following machine instructions into binary, use spaces to separate fields, and registers in their symbolic form ($ra NOT $31). Assume absolute jump addresses.

| Field 1 | Fields 2 and etc | instruction |
| --- | --- | --- |
| 001011 (0xb) | 11101 00001 0000 0000 0000 0011 ($sp) ($at) (3) | sltiu $at,$sp,3 |
| 000000 (0) | 0000000000 11111 00000 010000 (0) ($ra) (0) (0x10) | mfhi $ra |
| 000000 (0) | 00000 01001 10001 00000 100010 ($0) ($t1) ($s1) (0) (0x22) | neg $s1,$t1 (2’s complement s1= -t1;) sub $s1,$0,$t1 |
| 000010 (2) | 0000000000 0001 1110 1011 1111 (0x1ebf) = word offset | j 0x7afc (0x7afc = byte offset) |
| 101001 (0x29) | 11110 00010 0000 0000 0000 1100 ($fp) ($v0) (12) | sh $v0,12($fp) |
| 0x03e00008 000000 11111 0000 0000 0000 0000 01000 (0) ($ra) (0) (8) |  | jr $ra |

Problem 2 (7%).

Assume each part is independent. Assume absolute jump & branch addresses (no pc relative).

Fill in only registers that changed!

What is the value of the register or memory contents after the execution of the instruction.

Assume  pc = 200;   $s3=13;  $s4=6;  $ra=250; memory[8]=0xfedcba98; memory[12]=0x76543210;

| Instruction | pc | $ra | $s3 | $s4 | memory[8] | Memory[12] |
| --- | --- | --- | --- | --- | --- | --- |
| srl $s3, $s4, 3 | 204 |  | 0 |  |  |  |
| lui $s3, 0xffff | 204 |  | 0xFFFF0000 |  |  |  |
| lh $s4, 8($s4) | 204 |  |  | 0x3210 |  |  |
| addi $s3, $s3,4 | 204 |  | 9 |  |  |  |
| sb $s4, 7($s4) | 204 |  |  |  |  | 0x76063210 |
| bnez $ra, 600 | 600 |  |  |  |  |  |
| j 250 | 250 |  |  |  |  |  |

Problem 3. (25%) Translate the following C code into MIPS. Please comment your code. Assume signed unless defined otherwise. The value x is $s0; the value y is $s1; the value s is $s2; t is $s3; the address of r is in $s4; p is $s5; the address of d is in $s6, w is $s7.

Points will be taken off for assembler syntax errors.

register unsigned int x, y; register int s, *t; struct { int a[3]; short b; } r, *p; short d[5]; char *w;

- s  = *t;

lw  $s2, 0($s3)	#s= (int) *t

- s = *w;

lb $s2, 0($s7)		#s = (char) *w

(c)   s += d[x + 3];

addiu $t0, $s0, 3 	#$t0=x+3

addu  $t1, $t0, $t0	#$t1=(x+3)*sizeof(short) = (x+3)*2

addu  $t3, $s6, $t1	#$t3={address of d+(x+3)*sizeof(short)}

lh    $t4,  0($t3)  #$t4= (short) d[x+3]

add   $s2, $s2, $t4 #s = (signed int) {s + d[x+3]}

(d)  *(d  +  3) = s + r.b;  /* offsets within struct  0 : a[0], 4: a[1], 8:a[2], 12 : b */

lh $t0, 12($s4) 	#$t0= (short) r.b

add $t1, $s2, $t0	#$t1= (signed int) {s+r.b}

sh  $t1, 6($s6)     #*(d+3)= (short)d[3]{6=3*sizeof(short)}

(e)  for(x=0;   x <= 10;   x++ )   {  s  = (s >> 13); }   /* or {s >>= 13} */

addi  $t0, $0, 10	#$t0 = 10

addu  $s0, $0, $0	#(unsigned int ) x =0

L2: 	bgt   $s0, $t0,L1	#if ( x > 10) goto L1

sra   $s2, $s2,13	#(signed int)s >> 13

addiu $s0, $s0, 1	#(unsigned int) x++;

j L2

L1:

Problem 4. (25%)  Translate the following code and add comments

Points will be taken off for assembler syntax errors.

int	unicodestrlen(short *s)  {

register int  n = 0;

for(i=0;  s[i] != 0;  i++) {n++; }

return n;

}

(a) Write the prolog

#prolog is empty because:

• $t0..$t4 registers are only used (by convention these registers

are not required to be saved).

• Function does not call another function, therefore no need to

save $ra.

(b)  Write the body (hint: write the body first; then write the prolog)

addi $t0,$0,0		#n=0;

addi $t1,$0,$0		#i=0;

add $t2,$t1,$t1

L2:	sll  $t2,$t1,1		#$t2 = i * sizeof(short)

add  $t3,$a0,$t2	#$t3 = address(s + i)

lh   $t4,0($t3)	#$t4 = *(s+i) = s[i]

beq  $t4,$0,L1

addi $t0,$t0,1		#n++;

addi $t1,$t1,1		#i++;

j L2

L1:  addi $v0,$t0,0		#return n

(c) Write the epilog

END: jr $ra			#return

Alternative Solution

addi  $v0, $0, 0

L2:  lh    $t4, 0($a0)

beq   $t4, $0, END

addi  $v0, $0, 1

addi  $a0, $0, 2 /*2 = sizeof(short)*/

j L2

END: jr $ra

Problem 5. (10%)  Translate the following global variables and assign the location counter beginning at 700

- class xxstring {

char	argv;

short	montab[2];

short	(*daytab)[13];

void	(*strcpy)();

class	xxstring **next;

} *fsp;

| Decimal Location | Assembler definitions |
| --- | --- |
| 700 | fsp : .word 0 |
| 704 |  |

- class ccstring {

char	argv;

short	montab[2];

short	(*daytab)[13];

void	(*strcpy)();

class	ccstring **next;

} *fcp;

| Decimal Location | Assembler definitions |
| --- | --- |
| 700 | fcp : .word 0 |
| 704 |  |

Problem 6. (15%)  Given the following instruction sequence in the table below.

Assume the (alu and slt instructions are 5 clocks); (loads 10 clocks); (stores 20 clocks); (jumps 2 clocks); (branches 3 fall through/6 for branch);

(a)  Show the best case timing path through the code showing annotations and total.

(b)  Show the worst case timing path through the code showing annotations and total.

(c)  What values for $s1,$s2,$s3,$s4 will make this code execute the worst case?

$s1 < $s2 ; $s3,$s4 any values

| Instruction | best case | worst case |
| --- | --- | --- |
| slt $t1,$s1,$s2 | 5 | 5 |
| bne $t1,$0,L1 | 3 | 6 |
| addi $t2,$zero,5 | 5 | X |
| j L2 | 2 | X |
| L1: slt $t2,$s3,$s4 | X | 5 |
| beq $0,$0,L2 | X | 6 |
| xori $s3,$s4,3 | X | X |
| L2: addi $s1,$zero,10 | 5 | 5 |
| Total Time | 20 | 27 |