| EECS 314 | # Test 3 solutions | ## Wednesday April 3, 2002 |
| --- | --- | --- |

| Name: |  | ## Email: ______________________________ |
| --- | --- | --- |

Problem 1: A group of EECS students have decided to compete with Motorola Corporation in the embedded DSP wireless network market. The RISCEE3 computer is a 16 bit single-cycle computer.

There is only 1 register (i.e. accumulator, called A). The PC and alu are eight bits wide.

Note: You can remove and keep the RISCEE3 and RISCEE4 diagrams from the exam.

Remember: anything AND with zero is always zero. Anything OR with one is one.

There is only one instruction format shown as follows:

| Opcode 8 bits 15-8 | Data8 or Address8 field 8 bits 7 - 0 |
| --- | --- |

- (XX%) Fill in the settings of the control lines determined by the all the instructions (use X for Don’t Care)

{ See architecture solutions on powerpoint slides }

| Machine Instruction | Operation | RegDst | ALU | MemWrite | RegWrite | BZ | P0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| clear | A = 0 | 2 | 4 | 0 | 1 | 0 | 0 |
| addi data8 | A = A + data8 | 2 | 5 | 0 | 1 | 0 | 0 |
| add addr8 | A = A + Memory[addr8] | 0 | 3 | 0 | 1 | 0 | 0 |
| store addr8 | Memory[addr8] = A | X | 3 | 1 | 0 | 0 | 0 |
| bne addr8 | If (A != 0) { PC = addr8;} | X | 1 | 0 | 0 | 1 | 0 |
| apc | A = PC+2 | 3 | X | 0 | 1 | 0 | 0 |

(b) (XX%) Using the above instruction set, fill in the code for the pseudo-instructions

| Pseudo-Instruction | Operation | Assembly machine instructions (from part 1a) |
| --- | --- | --- |
| loadi data8 | #### A = data8 | clear # A=0; addi data8 # A=0+data8 |
| load address8 | A = Memory[addr8] | clear # A=0; add addr8 # A=0+Memory[addr8] |
| jmp address8 | pc = address8 | clear # A=0; addi 1 # A=1; always not zero bne addr8 # if (1 != 0) { pc = addr8; } #alternate solution: apc #A=PC; Assume PC not zero bne addr8 #always branch |
| jal address8 | A=PC+x; PC=addess8 Where A really contains the return address after the pseudo instruction returns. (Assume PC never becomes zero) | apc #A = PC+2: addi 4 #…+ length of addi and bne bne addr8 #return address here (=A)! |

Problem 2: RISCEE4 is the multi-cycle architecture for the machine instructions of problem 1a. Use X for Don’t Care. Assume parts 2a, 2b, 2c are independent of each other. Assume the 8 bit memory system is smart and loads the proper 16 bits in the IR register in one memory read cycle.

{ See architecture solutions on powerpoint slides }

(a) (XX%) Fill in the settings of the control lines needed for the “clear” instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 0 | 1 | 1 | 1 | 1 | X | 1 | 5 | 0 | 0 | 0 | X | fetch |
| T2 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 0 | X | decode |
| T3 | 0 | 0 | X | 0 | 0 | 0 | X | 4 | X | X | X | X | Aluout=0 |
| T4 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 1 | 0 | ###### A=aluout |

T1 and T2: RegWrite could equal X here because clear instruction will overwrite it later in T4
	       but since are used by all other instructions also at T1 and T2, RegWrite must equal 0.

##### Alternate solution: Merge T2 & T3 from above. During instruction decode set ALUOut to zero

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 0 | 1 | 1 | 1 | 1 | X | 1 | 5 | 0 | 0 | 0 | X | fetch |
| T2 | 0 | 0 | X | 0 | 0 | 0 | X | 4 | X | X | 0 | X | Decode Aluout=0 |
| T3 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 1 | 0 | ###### A=aluout |

(b) (XX%) Fill in the settings of the control lines needed for “add” from memory instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 0 | 1 | 1 | 1 | 1 | X | 1 | 5 | 0 | 0 | 0 | X | fetch |
| T2 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 0 | X | decode |
| T3 | 0 | 0 | X | 0 | 0 | 0 | X | 1 | X | 3 | 0 | X | ###### Aluout= IR[7:0] |
| T4 | 0 | 1 | 0 | 0 | 0 | 0 | X | X | X | X | 0 | X | ###### Memread |
| T5 | 0 | 0 | X | 0 | 0 | 0 | X | 5 | 1 | 1 | 0 | X | ALUout=mdr+a |
| T6 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 1 | 0 | ###### A=aluout |

##### Alternate solution: Merge T2 & T3 from above. During instruction decode set ALUOut to addr8

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 0 | 1 | 1 | 1 | 1 | X | 1 | 5 | 0 | 0 | 0 | X | fetch |
| T2 | 0 | 0 | X | 0 | 0 | 0 | X | 1 | X | 3 | 0 | X | Decode Aluout= IR[7:0] |
| T3 | 0 | 1 | 0 | 0 | 0 | 0 | X | X | X | X | 0 | X | ###### Memread |
| T4 | 0 | 0 | X | 0 | 0 | 0 | X | 5 | 1 | 1 | 0 | X | ALUout=mdr+a |
| T5 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 1 | 0 | ###### A=aluout |

(c) (XX%) Fill in the settings of the control lines needed for “bne” instruction

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | 0 | 1 | 1 | 1 | 1 | X | 1 | 5 | 0 | 0 | 0 | X | fetch |
| T2 | 0 | 0 | X | 0 | 0 | 0 | X | X | X | X | 0 | X | decode |
| T3 | 0 | 0 | X | 0 | 0 | 1 | 2 | 1 | 1 | X | 0 | X | ###### PC= (A!=0)? IR[7-0]; |

(d) (XX%) Fill in the critical path times for each instruction. The delay time of the functional units are as follows Memory Write 8 ns, Memory Read 5 ns, Register (read or write) and opcode decode 1 ns, and ALU  & Adders 2 ns.

| #### Instruction | Instruction memory | Decode & Register Read | 1st ALU operation | Data Memory | 2nd ALU operation | Register Write | Total Time | Clock Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clear | 5 | 1 | 2 |  |  | 1 | 9 | 4 |
| alternate clear | 5 | 1 |  |  |  | 1 | 7 | 3 |
| addi | 5 | 1 | 2 |  |  | 1 | 9 | 4 |
| add | 5 | 1 | 2 | 5 | 2 | 1 | 16 | 6 |
| store | 5 | 1 | 2 | 8 |  |  | 16 | 4 |
| bne | 5 | 1 | 2* |  |  |  | 8 | 3 |
| apc | 5 | 1 | 2 |  |  | 1 | 9 | 4 |

* Branch needs 2ns to compute zero value detect in ALU.

(e) (XX%) Determine the fastest clock speed for the computer to work properly in frequency and show why.

{ Graders: use the slowest resource from part 2d in columns 2 to 7 }

Clock period is the slowest resource in any one step: 8 ns

Clock frequency = 1/period = 1/8ns = 125 Mhz

(f) (XX%) Fill in the Clock, CPI, and MIPS in the above table and show all calculations.

{ Graders: use the student’s own data from part 2d and 2e. Grade only CPI and MIPS }

| Instruction | Clock Cycles | Instruction Mix |  |  |
| --- | --- | --- | --- | --- |
| clear | 4 | 10% |  |  |
| addi | 4 | 30% |  |  |
| add | 6 | 20% |  |  |
| store | 4 | 10% |  |  |
| bne | 3 | 5% |  |  |
| apc | 4 | 25% |  |  |
| Clock speed | 125 MHz |  |  |  |
| CPI | 4.35 | 4*(10%+30%+10%+25%)+3*5%+6*20%=4*75%+3*5%+6*20% |  |  |
| MIPS | 28.7 | 125 MHz/4.35 |  |  |

Alternate solution: Clear	= 3 clocks
	CPI	= 4.25 = 4*(30%+10%+25%)+3*(10%+5%)+6*20% 
	MIPS	= 29.4 = 125MHz/4.25

Problem 3.  Assume a simple 6 stage pipeline with the following execution times

| 1 | IF | Instruction fetch 1st part | 3 ns |
| --- | --- | --- | --- |
| 2 | IS | Instruction fetch 2nd part | 4 ns |
| 3 | ID | Register Read | 2 ns; Branch decision made here |
| 4 | EX | ALU | 3 ns |
| 5 | MEM | Data Access | 8 ns, |
| 6 | WB | Register Write | 2 ns |

This computer has the following instructions:

| Instruction | Operation |
| --- | --- |
| add $rd, $rs, $rt | $rd = $rs + $rt |
| beq $rs, $rt, disp16 | pc = pc+2+($rs-$rt=0?disp16:0) |
| lw $rt, addr16($rs) | $rt = Mem[addr16+$rs] |
| sw $rt, addr16($rs) | Mem[addr16+$rs]=$rt |

3a (XX%) Fill in the following tables

| Instruction | IF | IS | ID | EX | MEM | WB | Total Time | Multi- Cycles | Instruction Mix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add | 3 | 4 | 2 | 3 |  | 2 | 14 | 5 | 50% |
| beq | 3 | 4 | 2 |  |  |  | 9 | 3 | 25% |
| lw | 3 | 4 | 2 | 3 | 8 | 2 | 22 | 6 | 17% |
| sw | 3 | 4 | 2 | 3 | 8 |  | 20 | 5 | 8% |

3b (XX%) Fill the following table and show work.

| Instruction | Clock frequency | CPI (no hazards) | MIPS (no hazards) |
| --- | --- | --- | --- |
| Single-cycle CPU | 1/22ns =45.5 MHz | =1 | 45.5/1 = 45.5 MIPS |
| Multi-cycle CPU | 1/8ns =125 MHz | 5*50% + 3*25% + 6*17% + 5*8% =4.67 | 125/4.67 =26.8 MIPS |
| Pipeline CPU | 1/8ns =125 MHz | (no hazards = no dependancies = no stalls) =1 | 125/1 =125 MIPS |

The following parts refer to the pipelined machine only

For the following code: Assume no forwarding.

3c (XX%) For the following code: Assume no forwarding and no branch prediction.

Draw lines showing all the data dependencies and show the pipeline sequence (IF,IS,ID,EX,M,WB)

Note: Branch decision is made in the ID stage.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub $3, $4, $5 | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| beq $1, $3, loop |  | IF | IS | ID | ID | ID | EX | M | W |  |  |  |  |  |  |  |
| add $4, $4, $5 |  |  | IF | IS | IS | IS | ID | EX | M | W |  |  |  |  |  |  |

3d (XX%) Draw lines showing all the data dependencies in “Time” column.

and show the 6-stage pipeline sequence (IF, IS, ID, EX, M, WB) for the following code

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sw $1, 4($2) | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| sub $4, $1, $2 |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |
| add $5, $4, $2 |  |  | IF | IS | ID | ID | ID | EX | M | W |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  | IF | IS | IS | IS | ID | ID | ID | EX | M | W |  |  |  |

3e (XX%) Using forwarding, show the 6-stage pipeline sequence (IF, IS, ID, EX, M, WB)

and draw lines showing the forwarding.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sw $1, 4($2) | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| sub $4, $1, $2 |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |
| add $5, $4, $2 |  |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |

3f (XX%) Increase the number of pipeline stages to 7 stages. What stage would you split and why?

I would split the MEM stage into 2 stages (Mem1=4ns and Mem2=4ns) since is the slowest resource (8ns) of all the others.
Note: any other combination would not be optimal: Mem1=3ns and Mem2=5ns

Decreasing the worst case stage delay allows for the pipeline clock to increase.

This results in a faster MIPS.

3g (XX%) What is the 7-stage pipeline clock? 4ns or 250 MHz and (assuming no hazards) MIPS= 250 MIPS

MIPS = Clock/CPI = 250 Mhz / 1CPI

Note: 250 MIPS is now faster than all cases in problem 2b!

Problem 4. Show all calculations for the following questions.

Assume an add takes 1 cycle if no dependency and if dependant then 3 clocks

Assume there is a 30% data dependency.

Assume a branch takes 2 cycle if true prediction and if false prediction then 7 clocks.

Assume that 15% of the branches are mispredicted.

4a (XX%) What is the average add instruction time in clocks? __1.6___

1*70% + 3*30% = 1.6

4b (XX%) What is the average branch instruction time in clocks? ___2.75___

2*85% + 7*15% = 2.75