| EECS 314 | # Test 3 | ## Monday April 8, 2002 |
| --- | --- | --- |

| Name: |  | ## Email: ______________________________ |
| --- | --- | --- |

Problem 1. Show all calculations for the following questions.

Assume an add takes 2 cycle if no dependency and if dependant then 4 clocks

Assume there is a 25% data dependency.

Assume a branch takes 7 cycle if true prediction and if false prediction then 2 clocks.

Assume that 10% of the branches are mispredicted.

1a (5%) What is the average add instruction time in clocks?

1b (5%) What is the average branch instruction time in clocks?

Problem 2.  Assume a simple 6 stage pipeline with the following execution times

| 1 | IF | Instruction fetch | 5 ns |
| --- | --- | --- | --- |
| 2 | ID | Register Read 1st part | 1 ns |
| 3 | IR | Register Read 2nd part | 2 ns; |
| 4 | EX | ALU | 4 ns Branch decision made here |
| 5 | MEM | Data Access | 6 ns, |
| 6 | WB | Register Write | 3 ns |

This computer has the following instructions:

| Instruction | Operation |
| --- | --- |
| sub $rd, $rs, $rt | $rd = $rs  $rt |
| beq $rs, $rt, disp16 | pc = pc+2+($rs-$rt=0?disp16:0) |
| lw $rt, addr16($rs) | $rt = Mem[addr16+$rs] |
| sw $rt, addr16($rs) | Mem[addr16+$rs]=$rt |

2a (20%) Fill in the following tables

| Instruction | IF | ID | IR | EX | MEM | WB | Total Time | Multi- Cycles | Instruction Mix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub |  |  |  |  |  |  |  |  | 30% |
| beq |  |  |  |  |  |  |  |  | 10% |
| lw |  |  |  |  |  |  |  |  | 40% |
| sw |  |  |  |  |  |  |  |  | 20% |

2b (15%) Fill the following table and show work.

| Instruction | Clock frequency | CPI (no hazards) | MIPS (no hazards) |
| --- | --- | --- | --- |
| Single-cycle CPU |  |  |  |
| Multi-cycle CPU |  |  |  |
| Pipeline CPU |  |  |  |

The following parts refer to the pipelined machine only

2c (15%) For the following code: Assume no forwarding and no branch prediction.

Draw lines showing all the data dependencies and show the pipeline sequence (IF,ID,IR,EX,M,WB)

Note: Branch decision is made in the EX stage.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw $3, 0($4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| beq $1, $3, loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sub $4, $4, $5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Problem 3: The RISCEE5 computer is a multi-cycle architecture for the machine instructions of problem 1a. Use X for Don’t Care. Assume parts a, b, c, and d are independent of each other. Assume the 8 bit memory system is smart and loads the proper 16 bits in the IR register in one memory read cycle. There is only a single 8 bit register (i.e. accumulator, called A). The PC and alu are also eight bits wide.

Note: You can remove and keep the RISCEE5 diagrams from the exam.

Remember: anything AND with zero is always zero. Anything OR with one is one.

There is only one instruction format shown as follows:

| Opcode 8 bits 15-8 | Data8 or Address8 field 8 bits 7 - 0 |
| --- | --- |

| Machine Instruction | Operation | Description |
| --- | --- | --- |
| clear | A = 0 | Clear accumulator |
| load addr8 | A = Memory[addr8] | Load accumulator from memory |
| jptr addr8 | PC = Memory[addr8] | Jump to memory pointer |
| sll3 | A = A << 3 | Shift Left Logical accumulator by three |

3a (5%) Fill in the settings of the control lines needed for the “clear” instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | MDR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | Reg Dst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |  |

3b (10%) Fill in the settings of the control lines needed for “load” from memory instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | MDR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | Reg Dst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

3c (10%) Fill in the settings of the control lines needed for “jptr” instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | MDR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | Reg Dst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

3d (15%) Fill in the settings of the control lines needed for “sll3” instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | MDR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | Reg Dst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T7 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T8 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T10 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Extra Credit (25%): For x1, x2, and x3: refer to Problem 2 for pipeline description.

x1 (5% Using no forwarding, Draw lines showing all the data dependencies in “Time” column.

and show the 6-stage pipeline sequence (IF, ID, IR, EX, M, WB) for the following code

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub $4, $1, $2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sw $4, 4($2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sub $5, $4, $2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

x2 (5%) Using no forwarding, Draw lines showing all the data dependencies in “Time” column. (hint: “sw” instruction has a new register dependancy) and show the 6-stage pipeline sequence (IF, ID, IR, EX, M, WB) for the following code

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub $4, $1, $2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sw $4, 4($4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sub $5, $4, $2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

x3 (5%) Using forwarding, Draw lines showing all the data dependencies in “Time” column. (hint: “sw” instruction), and show the 6-stage pipeline sequence (IF, ID, IR, EX, M, WB) for the following code and draw lines showing the forwarding.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub $4, $1, $2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sw $4, 4($4) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sub $5, $4, $2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

x4 (10%) Refer to Problem 3. Remove the datapath between ALU and the PCSrc mux selection number “1”.  Fill in the settings of the control lines needed for the “clear” instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | MDR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | Reg Dst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T6 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T7 |  |  |  |  |  |  |  |  |  |  |  |  |  |