| EECS 322 | # Test 1 | ## Wednesday February 14, 2001 |
| --- | --- | --- |

| Name: |  |  |
| --- | --- | --- |

Problem 1 (18%). Assemble the following machine instructions into binary, use spaces to separate fields, and registers in their symbolic form ($ra NOT $31). Assume absolute jump addresses.

| Field 1 | Fields 2 and etc | instruction |
| --- | --- | --- |
| 000000 | 00011 00000 00000 00000 001000 $3 | jr $v1 jr $3 |
| 001101 | 00000 01010 0000 0000 1101 1110 $0 $10 0xde | ori $10, $0, 0xde |
| 101000 page A-67 | 11101 00000 0000 0000 0001 0001 $29 $0 17 | sb $zero,17($sp) sb $0,17($29) |
| 000100 | 10011 11001 0000 0111 0100 0001 $19 $25 0x741 | beq $s3,$t9, 0x741 beq $19,$25, 0x741 |
| 000000 | 00000 00000 11111 00000 100000 $0 $0 $31 | clear $ra add $31,$0,$0 |
| 000000 | 0 0 0 0 0 0 0 0 1 0 1 1 1 1 1 0 0 0 1 1 0 0 0 0 1 0 x x x x x v v v v v r r r r r 3 3 3 3 3 x x x x x x | srl $ra,$v0,3 |

Problem 2 (7%).

Assume each part is independent. Assume absolute jump & branch addresses (no pc relative).

Fill in only registers that changed!

What is the value of the register or memory contents after the execution of the instruction.

Assume  pc = 2020;   $s3=12;  $s4=4;  $ra=250;  memory[12]=0x18701914;

| instruction | pc | $ra | $s3 | $s4 | memory[8] | memory[12] |
| --- | --- | --- | --- | --- | --- | --- |
| jr $31 same as jr $ra | 250 |  |  |  |  |  |
| and $s3, $s3, $zero | 2024 |  | 0 |  |  |  |
| sw $s4, 8($s4) | 2024 |  |  |  |  | 4 |
| pseudo-instructionmachine instruction li $reg,small16addi $reg,$0,small16 li $reg,big32ori $reg,$reg,lower_half(big32) lui $reg,upper_half(big32) li $s3,0x17761812 | 2028 |  | 0x17761812 |  |  |  |
| slt $s3, $s3, $s4 | 2024 |  | 0 |  |  |  |
| bne $s3,$s4,40 | 40 |  |  |  |  |  |
| li $s4,0x1776 | 2024 |  |  | 0x1776 |  |  |

Problem 3. (25%) Translate the following C code into MIPS. Please comment your code.

Assume x is $t2; y is $t6; p is $a1 and points to integers; s is $a3 and points to unsigned char.

No pseudo-assembler instructions allowed.   Points will be taken off for assembler syntax errors.

- x  = (x & y ) + (y << 5);

and	$t0,$t2,$t6		# t0=x & y

sll	$t1,$t6,5			# t1=y << 5

add	$t2,$t0,$t1		# x= t0 + t1

(b)  x  =  (y <= 3)?  0x1999 : y;				# !(y  3)  (y ! 3)  y>3  3<y

#  bne(3<y)

addi	$t0,$0,3			# t0=3

slt	$t1,$t0,$t6		# if (3 < y ) t1=1; else t1=0;

bne	$t1,$0,L1			# if (t1 != 0) goto L1

addi	$t2,$0,0x1999		# then x = 0x1999

j	L2

L1:	add	$t2,$0,$t6		# else x = y

L2:

(c)  for(x=y;   x > y;   x++ )   {  y = y – 3;  }		# !(x>y)  !(y<x)  beq(y<x)

add	$t2,$t6,$0		#initialize x=y;

L2:	slt	$t0,$t6,$t2		#if (y < x) t1=1; else t1=0;

beq	$t0,$0,L1			#if (t0 = = 0) goto L1

addi	$t6,$t6, – 3		#y = y –  3

addi	$t2,$t2,1			#x ++

# alternative solution (best)

lw$t0,12($a1)# t0=*((char )p +3*4)

sb$t0,3($a3)# *(s+3) = t0

j	L2

L2:

(d)   s[3] = p[3];

addi	$t0,$0,3		# byte offset  = int offset * sizeof(int)

add	$t1,$t0,$t0	# t1 = t0 * 2

add	$t3,$t1,$t1	# t3 = t1 * 2  (why did I not use $t2 as a temp?)

add	$t4,$a1,$t3	# t4 = p + t3 = p + t0*4 = p + 3*4

lw	$t5,0($t4)	# t5 = *t4 = *(p+t3)

add	$t7,$a3,3		# t7 = s + 3

sb	$t5,0($t7)	# *t7 = t5

#alternative solution (best)

#

add$t0,$a3,$t2# t1 = s + x

lbu$t0,2($t0)# t0 = *(t0+2)

addi$t0,$t0,2# t0 += 2

#

sll$t1,$t6,2# t1 = y<<2 = 4*y

add$t1,$a1,$t1# t1 = p + t1

sw$t0,0($t1)# *t1 = t0;

(e)  *(p + y)  =  *(s + 2 + x) + 2;

addi	$t0,$t2,2		# t0 = x + 2

add	$t1,$a3,$t0	# t1 = s + t0

lbu	$t3,0($t1)	# t3 = *t1

addi	$t3,$t3,2		# t3 += 3

add	$t4,$t6,$t6	# t4 = 2*y

add	$t5,$t4,$t4	# t5 = 4*y

add	$t7,$a1,$t5	# t7 = p + t5

sw	$t3,0($t7)	# *t7 = t3;

Problem 4. (25%)  Translate the following code and add comments

No pseudo-assembler instructions allowed.   Points will be taken off for assembler syntax errors.

void	f(int x, int y)  {

register int  w=0x1960;

w = f(y, x + 5);

if (y < x )  { return w; }

return x;

}

- Write the prolog

.text

f:	addi	$sp,$sp, -16	#allocate space for w, and $ra

sw	$ra,12($sp)

sw	$s2,8($sp)	# needed for w

sw	$s1,4($12)	# needed for $a1 on recursion

sw	$s0,0($sp)	# needed for $a0 on recursion

addi	$s2,$0,0x1960	# w = 0x1960

- Write the body

Remember $a0 and $a1 are destroyed by the previous recursive jal.

Must use $s0, $s1 or restore by a load.

add	$s0,$a0,$0	# save argument x in $s0

add	$s1,$a1,$0	# save argument y in $s1

add	$a0,$s1,$0	# $a0 = y

add	$a1,$s0,5		# $a1 = x + 5

jal	f			# $v0 = f($a0,$a1)

# x and y cannot be $a0 & $a1 since they as not saved

# !(x < y)  beq(x < y)

slt	$t0,$s0,$s1	# if (y < x) t0=1; else t0=0;

beq	$t0,$0,L1		# if (t0 = = 0 ) goto L1;

add	$v0,$v0,$0	# return w = return of jal f

j	f_epilog

L1:	add	$v0,$s0,$0	# return x;

# jump or fall through to epilog

(c) Write the epilog

f_epilog:

lw	$s0,0($sp)	#restore caller

lw	$s1,4($sp)

lw	$s2,8($sp)

lw	$ra,12($sp)

addi	$sp,$sp,16	#must match entry allocation

jr	$ra

Problem 5. (10%)  Translate the following global variables

x: name of variable is the label

- int x[3] = { 0x1920, 0x1930 };

.data

x:	.word	0x1920, 0x1930, 0	# “3” words allocated

# BUT only “2” are  initialized!

- struct keyword {

short	*p,  x;

unsigned char    t[2];

struct treenode  *left;

}   fp;

.data

fp:						# NECESSARY LABEL for accessing struct!

fp_p:	.word	0		# short *p;

fp_x:	.half	0		# short x;

fp_t:	.byte	0,0		# unsigned char t[2];

fp_left:	.word	0		# struct treenode *left;

Problem 6. (15%)  Given the following instruction sequence in the table below.

Assume the (alu and slt instructions are 5 clocks); (loads 10 clocks); (stores 20 clocks); (jumps 2 clocks); (branches 5 fall through/10 for branch);

(a)  Show the best case timing path through the code showing annotations and total. #all 4 paths

(b)  Show the worst case timing path through the code showing annotations and total. #all 4 paths

(c)  What values will make this code execute the worst case?

# Must state $a0,$a1, $a2. Any other register is wrong!

$a0 = any value, $a1= any value, $a2=any value

| instruction | best case | worst case |
| --- | --- | --- |
| slt $t0,$a0,$a1 | 5 5 5 5 | 5 5 5 5 |
| bne $t0,$zero,L1 | 10 10 5 5 | 10 10 5 5 |
| addi $t2,$zero,5 | 5 5 | 5 5 |
| L1: beq $a2,$zero,L2 | 10 5 10 5 | 10 5 10 5 |
| addi $a1,$a1,3 | 5 5 | 5 5 |
| L2: addi $s1,$zero,10 | 5 5 5 5 | 5 5 5 5 |
| Total Time | 30 30 30 30 | 30 30 30 30 |