| EECS 322 | # Post Test 2 (12 points) | ## Due Monday April 2, 2001 |
| --- | --- | --- |

| Name: |  | ## Email: ______________________________ |
| --- | --- | --- |

Problem 1: A group of EECS students have decided to compete with Motorola Corporation in the embedded DSP wireless network market. The RISCEE3 computer is a 16 bit single-cycle computer.

There is only 1 register (i.e. accumulator, called A). The PC and alu are eight bits wide.

Note: You can remove and keep the RISCEE3 and RISCEE4 diagrams from the exam.

Remember: anything AND with zero is always zero. Anything OR with one is one.

There is only one instruction format shown as follows:

| Opcode 8 bits 15-8 | Data8 or Address8 field 8 bits 7 - 0 |
| --- | --- |

(a) Fill in the settings of the control lines determined by the all the instructions (use X for Don’t Care)

| Machine Instruction | Operation | RegDst | ALU | MemWrite | RegWrite | BZ | P0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| clear | A = 0 |  |  |  |  |  |  |
| addi data8 | A = A + data8 |  |  |  |  |  |  |
| add addr8 | A = A + Memory[addr8] |  |  |  |  |  |  |
| store addr8 | Memory[addr8] = A |  |  |  |  |  |  |
| bne addr8 | If (A != 0) { PC = addr8;} |  |  |  |  |  |  |
| apc | A = PC+2 |  |  |  |  |  |  |

(b) Using the above instruction set, fill in the code for the pseudo-instructions

| Pseudo-Instruction | Operation | Assembly machine instructions (from part 1a) |
| --- | --- | --- |
| loadi data8 | #### A = data8 |  |
| load address8 | A = Memory[addr8] |  |
| jmp address8 | pc = address8 |  |
| jal address8 | A=PC+x; PC=addess8 Where A really contains the return address after the pseudo instruction returns. (Assume PC never becomes zero) |  |

Problem 2: The Credit Suisse First Boston investment bank will only invest in the multi-cycle RISCEE4 architecture for the machine instructions of problem 1a. Use X for Don’t Care. Assume parts 2a, 2b, 2c are independent of each other.  Do it in the minimum number of clocks.

(a) Fill in the settings of the control lines needed for the “clear” instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |

(b) Fill in the settings of the control lines needed for “add” from memory instruction.

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |
| T6 |  |  |  |  |  |  |  |  |  |  |  |  |

(c) Fill in the settings of the control lines needed for “bne” instruction

| Clock Step | MemWrite | MemRead | IorD | IR write | P0 | BZ | PC src | ALUop | ALU srcA | ALU srcB | Reg Write | RegDst |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |  |  |  |  |  |
| T4 |  |  |  |  |  |  |  |  |  |  |  |  |
| T5 |  |  |  |  |  |  |  |  |  |  |  |  |
| T6 |  |  |  |  |  |  |  |  |  |  |  |  |

Problem 3. Look up the the following patent

- US Patent 5153921 Microcomputer, 1992

using http://www.delphion.com with the search word as the patent number.

Note: use the View Images for the first free 7 pages

Check the web for patent infringement. For example, http://www.intellectual.com/patinfr.htm or

use http://www.google.com

- Briefly explain what claims (and sub items within a claim) match & don’t match the RISCEE 3 architecture?
Does RISCEE3 infringe on their patent?

- Briefly explain what claims (and sub items within a claim) match & don’t match the RISCEE 4 architecture?
Does RISCEE4 infringe on their patent?