| EECS 322 | # Test 3 | ## Wednesday April 18, 2001 |
| --- | --- | --- |

| Name: |  | ## email: _______________________________ |
| --- | --- | --- |

Problem 1 (20%).  Show each step of the pipeline machine (page 470 and 469) for the following instruction sequence: (note: Z is the zero flag from the ALU)

Assume $2=9, $4=8; Mem[12]=742; Mem[16]=1769. Treat the “nop” instruction as “add $0,$0,$0”.

or	$2, $4, $4		#opcode    $rd, $rs, $rt

sw	$4, 8($4)		#opcode    $rt, offset($rs)

nop

nop

| Clock | <IF/ID> <PC, IR> | <ID/EX> <WB,M,EX,PC,A,B,S,Rt,Rd> | <EX/MEM> <WB,M,PC,Z, ALU, B, R> | <MEM/WB> <WB,MDR,ALU,R> |
| --- | --- | --- | --- | --- |
| 0 | <0,?> | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 1 | <4,”or $2,$4, $4”> observe that or $rd=$2,$rs=$rt=$4 | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 2 | <8,”sw $4,8($4)”> observe that sw $rt=$4,8($rs=$4) | <10, 000, 1100, 4, 8, 8, X, $4,$2> observe that <wb=10, m=000, ex=1100, pc=4, fetch reg[$rs$4]8A, fetch reg[$rt$4]8B, S=00100 0000 100101=X, $rd=$4> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 3 | <12, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <0X, 001, X001, 8, 8, 8, 8, $4,$X> observe that <wb=0X ,m=001, ex=X001, pc=8, reg[$rs$4]8A, reg[$rt$4]8B, S=8, $rt=$4, $rd=$X> | <10, 000, X, 0, 8, 8, $2> observe that <wb=10, m=000, pc=X (don’t care, not a branch), ALU zero flag = Z=0, ALU=A or B = 8 + 8= 8, B=8, destination R = ($rt or $rd) = $2> | <?,?,?,?,?> |
| 4 | <16, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 12, 0, 0, 0, $0, $0> observe that <wb=10, m=000, ex=1100, pc=12, $rs=$0=0=A, $rt=$0=0=B, S=0, $rt=$0, $rd=$0> | <0X, 001, X, X or 0, 16, 8, X or $4> observe that <wb=0X, m=001, pc=X (don’t care, not a branch), ALU zero flag = Z = X or 0, ALU=A+offset = 8 + 8= 16, B=8, R = ($rt or $rd) = X or $4> (don’t care, no register writeback) ### Store to memory Mem[ALU=16]=8 | <10, X, 8, $2> Register writeback to $4 No memory fetch Observe that <wb=10, mdr=X, ALU=8, R=$2> |
| 5 | <20, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 16, 0, 0, 0, $0, $0> observe that <wb=10, m=000, ex=1100, pc=16, $rs=$0=0=A, $rt=$0=0=B, S=0, $rt=$0, $rd=$0> | <10,000,X,0,0,0,$0> observe that <wb=10, m=000, pc=X, Z=0, ALU=A+B=0+0=0, B=0, R=$0> | <0X, X, X, X or $4> Write to Memory |

Problem 2.  Assume a simple 6 stage pipeline with the following execution times

| 1 | IF | Instruction fetch 1st part | 3 ns |
| --- | --- | --- | --- |
| 2 | IS | Instruction fetch 2nd part | 4 ns |
| 3 | ID | Register Read | 2 ns; Branch decision made here |
| 4 | EX | ALU | 3 ns |
| 5 | MEM | Data Access | 8 ns, |
| 6 | WB | Register Write | 2 ns |

This computer has the following instructions:

| Instruction | Operation |
| --- | --- |
| add $rd, $rs, $rt | $rd = $rs + $rt |
| beq $rs, $rt, disp16 | pc = pc+2+($rs-$rt=0?disp16:0) |
| lw $rt, addr16($rs) | $rt = Mem[addr16+$rs] |
| sw $rt, addr16($rs) | Mem[addr16+$rs]=$rt |

2a (16%) Fill in the following tables

| Instruction | IF | IS | ID | EX | MEM | WB | Total Time | Multi- Cycles | Instruction Mix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add | 3 | 4 | 2 | 3 |  | 2 | 14 | 5 | 50% |
| beq | 3 | 4 | 2 |  |  |  | 9 | 3 | 25% |
| lw | 3 | 4 | 2 | 3 | 8 | 2 | 22 | 6 | 17% |
| sw | 3 | 4 | 2 | 3 | 8 |  | 20 | 5 | 8% |

2b (5%) Fill the following table and show work.

| Instruction | Clock frequency | CPI (no hazards) | MIPS (no hazards) |
| --- | --- | --- | --- |
| Single-cycle CPU | 1/22ns =45.5 MHz | =1 | 45.5/1 = 45.5 MIPS |
| Multi-cycle CPU | 1/8ns =125 MHz | 5*50% + 3*25% + 6*17% + 5*8% =4.67 | 125/4.67 =26.8 MIPS |
| Pipeline CPU | 1/8ns =125 MHz | (no hazards = no dependancies = no stalls) =1 | 125/1 =125 MIPS |

The following parts refer to the pipelined machine only

For the following code: Assume no forwarding.

2c (15%) For the following code: Assume no forwarding and no branch prediction.

Draw lines showing all the data dependencies and show the pipeline sequence (IF,IS,ID,EX,M,WB)

and draw lines showing the forwarding. Note: Branch decision is made in the ID stage.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub $3, $4, $5 | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| beq $1, $3, loop |  | IF | IS | ID | ID | ID | EX | M | W |  |  |  |  |  |  |  |
| add $4, $4, $5 |  |  | IF | IS | IS | IS | ID | EX | M | W |  |  |  |  |  |  |

2d (12%) Draw lines showing all the data dependencies in “Time” column.

and show the 6-stage pipeline sequence (IF, IS, ID, EX, M, WB) for the following code

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sw $1, 4($2) | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| sub $4, $1, $2 |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |
| add $5, $4, $2 |  |  | IF | IS | ID | ID | ID | EX | M | W |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  | IF | IS | IS | IS | ID | ID | ID | EX | M | W |  |  |  |

2e (12%) Using forwarding, show the 6-stage pipeline sequence (IF, IS, ID, EX, M, WB)

and draw lines showing the forwarding.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sw $1, 4($2) | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| sub $4, $1, $2 |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |  |
| add $5, $4, $2 |  |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  | IF | IS | ID | EX | M | W |  |  |  |  |  |  |  |

2f (5%) Increase the number of pipeline stages to 7 stages. What stage would you split and why?

I would split the MEM stage into 2 stages (Mem1=4ns and Mem2=4ns) since is the slowest resource (8ns) of all the others.
Note: any other combination would not be optimal: Mem1=3ns and Mem2=5ns

Decreasing the worst case stage delay allows for the pipeline clock to increase.

This results in a faster MIPS.

2g (5%) What is the 7-stage pipeline clock? 4ns or 250 MHz and (assuming no hazards) MIPS= 250 MIPS

MIPS = Clock/CPI = 250 Mhz / 1CPI

Note: 250 MIPS is now faster than all cases in problem 2b!

Problem 3. Show all calculations for the following questions.

Assume an add takes 1 cycle if no dependency and if dependant then 3 clocks

Assume there is a 30% data dependency.

Assume a branch takes 2 cycle if true prediction and if false prediction then 7 clocks.

Assume that 15% of the branches are mispredicted.

3a (5%) What is the average add instruction time in clocks? __1.6___

1*70% + 3*30% = 1.6

3b (5%) What is the average branch instruction time in clocks? ___2.75___

2*85% + 7*15% = 2.75

Over the past three years, the Goldman Sachs High Technology Group has been the lead manager for initial public offerings (IPOs) of 129 technology companies. They  (www.gs.com/hightech) would like you to have access to the following extra credit which can be used for this and previous exams.

For the following instruction sequence fill in the direct-mapped writeback data cache. The word size is 16 bits.   Memory[0]=$4=0x742; Memory[42]=$3=0x1412; Memory[52]=0x1585; Memory[58]=0x1769;

GS1a) (5%) Fill in the miss cache column.

| tag | index | byte offset | Instruction | Cache Miss? |
| --- | --- | --- | --- | --- |
| 111 | 01 | 0 | lw $1, 58($0) | Yes |
| 110 | 10 | 1 | lbu $2, 53($0) | Yes |
| 110 | 10 | 0 | sw $3, 52($0) | No (already loaded by lbu $2,53($0)) |
| 000 | 00 | 0 | lw $4, 0($0) | Yes |
| 101 | 01 | 1 | lbu $5, 43($0) | Yes |
| 111 | 01 | 0 | lw $6, 58($0) | Yes (flushed out by lbu $5,43($0)) |

GS1b) (10%) Show all states and underline the final state of the direct mapped data cache:

| Index | Valid | Dirty | Tag | Data |
| --- | --- | --- | --- | --- |
| 00 | NY | N | 000 | 0x742 |
| 01 | NY | N | 111101111 | 0x17790x14120x1769 |
| 10 | NY | NY | 110 | 0x15850x1412 |
| 11 | N | N |  |  |

GS2a. (10%) Assume 1536 bytes of real memory (0-511)(512-1023)(1024-1535); LRU, a page size of 512 bytes and no pages loaded in memory. Fill in the page fault columns. (Blank space implies No)

| instruction | Page fault? | Flush which real page? | Write flushed page to disk? | Load what new virtual page | Load into what real page |
| --- | --- | --- | --- | --- | --- |
| lw $1, 58($0) | Yes |  |  | 0 (=0..511) | 0 (=0..511) |
| sw $7, 52($0) |  |  |  |  |  |
| lw $5, 1412($0) | Yes |  |  | 2 (=1024..1535) | 1 (=512..1023) |
| lw $2, 742($0) | Yes |  |  | 1 (=512..1023) | 2 (=1024..1535) |
| sw $6, 1582($0) | Yes | 0 (=0..511) | Yes | 3 (=1526..2047) | 0 (=0..511) |
| sb $1, 43($0) | Yes | 1 (=512..1023) |  | 0 (=0..511) | 1 (=512..1023) |
| lbu $2, 1769($0) |  |  |  |  |  |

GS2b.. (5%) Fill out the TLB after execution of part GS2a. (hint: think of a fully associative cache)

| Valid | Dirty | Virtual Page Tag | Physical Page Number |
| --- | --- | --- | --- |
| NYN Y | NYN N | 0 | X0X1 |
| NYN | N NN | 2 | X1X |
| NY | N N | 1 | X2 |
| NY | N N | 3 | X0 |

Note: page 0 = 0..511, page 1 = 512..1023, page 2 = 1024..1535, page 3 = 1536..2047, …