| EECS 322 | # Test 4 solution | ## Monday April 30, 2001 |
| --- | --- | --- |

| Name: |  | ## email: _______________________________ |
| --- | --- | --- |

For the following instruction sequence fill in the direct-mapped writeback data cache. The word size is 32 bits.    Memory[0]=0x05370899; Memory[12]=0x10871625; Memory[60]=0x16581727;

Problem 1a. (25%) Fill in the miss cache column.

| tag | index | byte offset | Instruction | Cache Miss? |
| --- | --- | --- | --- | --- |
| 00 | 00 | 01 | lbu $6, 1($0) | YES |
| 11 | 11 | 10 | lw $1, 62($0) | YES |
| 00 | 00 | 11 | sb $0, 3($0) |  |
| 11 | 11 | 01 | lbu $2, 61($0) |  |
| 00 | 11 | 00 | lw $4, 12($0) | YES |
| 11 | 11 | 00 | sw $0, 60($0) | YES |

Problem 1b. (25%) Show all states and underline the final state of the direct mapped data cache:

| Index | Valid | Dirty | Tag | 32 bit Data |
| --- | --- | --- | --- | --- |
| 00 | NYY | XNY | X  00  00 | 0x05370899  0x05370800 |
| 01 | N | X |  |  |
| 10 | N | X |  |  |
| 11 | NYY | X  N  N  Y | X  11  00  11 | 0x16581727  0x10871625  0x00000000 |

P2a. (25%) Assume only 1024 bytes of real memory (0-511)(512-1023); LRU, a page size of 512 bytes and no pages loaded in memory. Fill in the columns. (Blank space implies No or none)

| Instruction virtual address | Page fault? | Flush which real page? | Write flushed page to disk? | Load what new virtual page | Load into what real page |
| --- | --- | --- | --- | --- | --- |
| lw $1, 742($0) | YES |  |  | 1 (512..1023) | 0 (0..511) |
| sw $2, 1412($0) | YES |  |  | 2 (1024..1535) | 1 (512..1023) |
| lbu $3, 1769($0) | YES | 0 (0..511) |  | 3 (1536..2047) | 0 (0..511) |
| sw $4, 814($0) | YES | 1 (512..1023) | YES | 1 (512..1023) | 1 (512..1023) |
| lw $5, 1431($0) | YES | 0 (0..511) |  | 2 (1024..1535) | 0 (0..511) |
| lbu $6, 1821($0) | YES | 1 (512..1023) | YES | 3 (1536..2047) | 1 (512..1023) |

P2b. (10%) After execution of part P2a: what virtual page is contained in the

real page 0 = 2  or  (1024..1535) and  real page 1 = 3  or  (1536..2047)

P2c. (15%) After execution of part P2a: fill out the TLB (Hint: think of a fully associative cache)

| Valid | Dirty | Virtual Page Tag | Physical Page Number |
| --- | --- | --- | --- |
| NYNYN | XNNYN | 1 or (512..1023) | X  0  X  1  X |
| NYNY | XYNN | 2 or (1024..1535) | X  1  X  0 |
| NYNY | XNNN | 3 or (1536..2047) | X  0  X  1 |

Note: page 0 = 0..511, page 1 = 512..1023, page 2 = 1024..1535, page 3 = 1536..2047, …

Robertson Stephens, typecast as the boutique bank specializing in Silicon Valley IPOs, underwrites IPOs, specializing in such high-tech issues as Sun Microsystems, Pixar Animation Studios, and E*TRADE. (Revenue per employee: $1,191,895.11)  www.robertsonstephens.com would like you to have access to the following extra credit which can be used for this and previous exams.

RS1 (10%) Using problem P2a, rewrite the instruction sequence which minimizes page faults.

Multiple solutions: 3 page faults or misses down from 6 page faults in P2a.

lw	$1, 742($0)

sw	$4, 814($0)

lbu	$3, 1769($0)

lbu	$6, 1821($0)

lw	$2, 1412($0)

lw	$5, 1431($0)

lw	$1, 742($0)

sw	$4, 814($0)

lbu	$3, 1769($0)

lw	$2, 1412($0)

lbu	$6, 1821($0)

lw	$5, 1431($0)

lw	$1, 742($0)

lw	$2, 1412($0)

sw	$4, 814($0)

lw	$5, 1431($0)

lbu	$3, 1769($0)

lbu	$6, 1821($0)

RS2 (10%). Assemble the following machine instructions into binary

| Field 1 =sw | $rs=$fp $rt=$s3 offset=4 | instruction |
| --- | --- | --- |
| 101011 | 11110 10011 0000 0000 0000 0100 Note: No divide by 4 on the immediate field. Only the branch & jumps require that. | sw $s3, 4($fp) |

RS3 (10%) Draw lines showing all the data dependencies in “Time” column. Using forwarding, show the 5-stage MIPS pipeline sequence (IF, ID, EX, M, WB) and draw lines showing the forwarding.

(Hint: sw $rt,offset($rs) when does a store word stall? $rt or $rs?)

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sub $2, $1, $4 | IF | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |  |
| sw $2, 4($1) |  | IF | ID | EX | M | W |  |  |  |  |  |  |  |  |  |  |
| add $5, $2, $1 |  |  | IF | ID | EX | M | W |  |  |  |  |  |  |  |  |  |
| lw $3, 8($5) |  |  |  | IF | ID | EX | M | W |  |  |  |  |  |  |  |  |

See  Page 481, Figure 6.37.

Page 489, sec 6.5: …one case where forwarding cannot save the day …a load instruction…

RS4. (20%) Translate the following C code into MIPS. Please comment your code. Assume w is $a1 and points to integers; cp is $a2 and points to unsigned char;   int x is $t3;  int y is $t4;.

Note: sw $rt,offset($rs) equals *($rs+offset)=$rt or $rs[offset]=$rs

(a)   cp[y] = x;

add	$t0, $a2, $t4		# $t0 = (char *cp)$a2 + (int y)$t4

sb	$t3, 0($t0)		# *(cp + y) = (int x)$t3

(b)   w[y] = x;

sll	$t0, $t4, 2		# $t0 = y << 2;

add	$t0, $a1, $t0		# $t0 = (int *w)$a1  +  y<<2;

sw	$t3, 0($t0)		# *(w + y) = (int x)$t3

#alternate solution

add	$t0, $t4, $t4		# $t0 = y*2;

add	$t0, $t0, $t0		# $t0 = (y*2)*2

add	$t0, $a1, $t0		# $t0 = (int *w)$a1  +  y<<2;

sw	$t3, 0($t0)		# *(w + y) = (int x)$t3