| EECS 322 | # Test 4 sample | ## Wednesday April 12, 2000 |
| --- | --- | --- |

| Name: |  | ## Note: This exam is longer than the actual one |
| --- | --- | --- |

Problem 1.  Show each step of the pipeline machine (page 540) for the following instruction sequence: Assume $1=9, $2=8; $3=5; $4=3; $5=2; Mem[12]=16

lw	$1, 4($2)

add	$3, $4, $5

nop

nop

| Clock | <IF/ID> <PC, IR> | <ID/EX> <WB,M,EX,PC,A,B,S,Rt,Rd> | <EX/MEM> <WB,M,PC,Z, ALU, B, R> | <MEM/WB> <WB,MDR,ALU,R> |
| --- | --- | --- | --- | --- |
| 0 | <0,?> | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

Problem 2.  Assume a simple 6 stage pipeline with the following execution times

| 1 | IF | Instruction fetch | 3 ns |
| --- | --- | --- | --- |
| 2 | ID | Register Read | 1 ns |
| 3 | EX | Multiply | 4 ns |
| 4 | EX2 | ALU | 2 ns |
| 5 | MEM | Data Access | 3 ns |
| 6 | WB | Register Write | 1 ns |

This computer has the following instructions:

| Instruction | Operation |
| --- | --- |
| add $rd, $rs, $rt | $rd = $rs + $rt |
| lw $rt, addr16($rs) | $rt = Mem[addr16+$rs] |
| sw $rt, addr16($rs) | Mem[addr16+$rs]=$rt |
| beq $rs, $rt, disp16 | pc = pc+2+($rs-$rt=0?disp16:0) |
| madd $rd, $rs, $rt | $rd = $rs*$rt + $rd |

(a) Fill in the following table

| Instruction | IM | ID | EX | EX2 | MEM | WB | Total Time | Multi- Cycles | Instruction Mix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add |  |  |  |  |  |  |  |  | 20% |
| lw |  |  |  |  |  |  |  |  | 20% |
| sw |  |  |  |  |  |  |  |  | 20% |
| beq |  |  |  |  |  |  |  |  | 20% |
| madd |  |  |  |  |  |  |  |  | 20% |

(b) Fill the following table

| Instruction | Clock frequency | CPI | MIPS |
| --- | --- | --- | --- |
| Single-cycle CPU |  |  |  |
| multi-cycle CPU |  |  |  |

The parts refer to pipelined machine only

(c) What is the instruction latency?

(d) What is the pipeline clock?

(e) What is the pipelined speed up?

(f) Without forwarding, fill in the hazard type (page 441-8) and condition (page 479), for the following instruction sequence:

| Instruction | Hazard type and condition |
| --- | --- |
| add $1, $2, $3 |  |
| sub $4, $2, $1 |  |
| lw $5, data($6) |  |
| beq $1, $5, loop2 |  |

(g) Without forwarding, draw the pipeline sequence (like figure 6.4) for part f

(h) .Show all timing, including stalls.

(i) Suppose the hazard detection unit does not work on the processor due to a bug in the design.  Using the nop instruction, rewrite the code of part f which will give correct results (like page 478).

(j) Using forwarding, draw graphical pipeline sequence (like figure 6.8) of part f. Show all shading.

(k) Show all timing including stalls.

(l) Using forwarding, reorder the code to minimize the stalling of part f.

(m) Given any 2 instruction combinations, what is the worst instruction sequence. (i.e. assume no forwarding or prediction). Draw the pipeline chart (like Figure 6.4). Determine the MIPS.

(n) Given any 2 instruction combinations, what is the worst instruction sequence using forwarding. Draw the pipeline chart (like Figure 6.4). Determine the MIPS.

(o) Using all the best possible instruction flow (i.e. no hazards, assume no forwarding or prediction) of the 5 instructions.  Determine the MIPS.

(p) Show the worst possible instruction pipeline sequence of using only the 5 instructions. (i.e. assume no forwarding or prediction). Draw the pipeline chart (Figure 6.4). Determine the MIPS.