| EECS 322 | # Test 4 sample | ## Wednesday April 12, 2000 |
| --- | --- | --- |

| Name: |  | ## Note: This exam is longer than the actual one |
| --- | --- | --- |

Problem 1.  Show each step of the pipeline machine (page 470 and 469) for the following instruction sequence: Assume $1=9, $2=8; $3=5; $4=3; $5=2; Mem[12]=16. Treat the “nop” instruction as “add $0,$0,$0”.

Clarification

lw	$1, 4($2)				lw	C$rtB, 4(C$rsA)

add	$3, $4, $5				add	C$rdR, C$rsA,C$rtB

nop						add	$rd=0, $rs=0, $rt=0

nop						note: register 0 is always zero!

| Clock | <IF/ID> <PC, IR> | <ID/EX> <WB,M,EX,PC,A,B,S,Rt,Rd> | <EX/MEM> <WB,M,PC,Z, ALU, B, R> | <MEM/WB> <WB,MDR,ALU,R> |
| --- | --- | --- | --- | --- |
| 0 | <0,?> | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 1 | <4,”lw $1,4($2)”> observe that lw $rt=$1,4($rs=$2) | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 2 | <8, “add $3,$4,$5”> observe that add $rd=$3,$rs=$4,$rt=$5 | <11, 010, 0001, 4, 8, 9, 4, $1,X> observe that <wb=11,m=010,ex=0001,pc=4, $rs$28A, $rt$19B, S=4, $rt=$1, $rd=X> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 3 | <12, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 8, 3, 2, X, $5, $3> observe that <wb=10,m=000, ex=1100, pc=8, $rs=$4=3=A, $rt=$5=2=B, S=X, $rt=$5, $rd=$3> | <11,010,20,0,12,9,$1> observe that <wb=11, m=010, pc=pc+(S<<2)=4+(4<<2)=20, Z=0, ALU=S+$2=4+8=12, 9, $1> | <?,?,?,?,?> |
| 4 | <16, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 12, 0, 0, 0, $0, $0> observe that <wb=10, m=000, ex=1100, pc=12, $rs=$0=0=A, $rt=$0=0=B, S=0, $rt=$0, $rd=$0> | <10,000,X,0,12,9,$3> observe that <wb=10, m=000, pc=X, Z=0, ALU=A+B=3+2=5, B=2, R=$3> | <11,16,12,$1> =>Reg[$1]=16 finally register 1 will contain the load memory word of 4 + $12 observe that <wb=11, mdr=Mem[ALU] =Mem[12]=16, ALU=12, R=$1> |
| 5 | <20, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 16, 0, 0, 0, $0, $0> observe that <wb=10, m=000, ex=1100, pc=16, $rs=$0=0=A, $rt=$0=0=B, S=0, $rt=$0, $rd=$0> | <10,000,X,0,0,0,$0> observe that <wb=10, m=000, pc=X, Z=0, ALU=A+B=0+0=0, B=0, R=$0> | <10,X,0,12,9,$3> =>Reg[$3]=5 finally register 3 will contain the add of $4 plus $5 in the writeback! observe that <wb=10, mdr=X, ALU=5, R=$3> |

Problem 2.  Assume a simple 6 stage pipeline with the following execution times

| 1 | IF | Instruction fetch | 3 ns |
| --- | --- | --- | --- |
| 2 | ID | Register Read | 1 ns |
| 3 | EX | Multiply | 4 ns |
| 4 | EX2 | ALU | 2 ns |
| 5 | MEM | Data Access | 3 ns |
| 6 | WB | Register Write | 1 ns |

This computer has the following instructions:

| Instruction | Operation |
| --- | --- |
| add $rd, $rs, $rt | $rd = $rs + $rt |
| lw $rt, addr16($rs) | $rt = Mem[addr16+$rs] |
| sw $rt, addr16($rs) | Mem[addr16+$rs]=$rt |
| beq $rs, $rt, disp16 | pc = pc+2+($rs-$rt=0?disp16:0) |
| madd $rd, $rs, $rt | $rd = $rs*$rt + $rd |

load & store requires alu to add addr16+$rs

(a) Fill in the following table

| Instruction | IM | ID | EX | EX2 | MEM | WB | Total Time | Multi- Cycles | Instruction Mix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add | 3 ns | 1 ns |  | 2 ns |  | 1 ns | 7 ns | 4 | 20% |
| lw | 3 ns | 1 ns |  | 2 ns | 3 ns | 1 ns | 10 ns | 5 | 20% |
| sw | 3 ns | 1 ns |  | 2 ns | 3 ns |  | 9 ns | 4 | 20% |
| beq | 3 ns | 1 ns |  | 2 ns |  |  | 6 ns | 3 | 20% |
| madd | 3 ns | 1 ns | 4 ns | 2 ns |  | 1 ns | 11 ns | 5 | 20% |

(b) Fill the following table

| Instruction | Clock frequency | CPI | MIPS |
| --- | --- | --- | --- |
| Single-cycle CPU | 1/11 ns= 90.9 MHz | 1 | 90.9 Mhz/1=90.9 MIPS |
| multi-cycle CPU | 1/4ns=250 Mhz | 4.2 | 250 MHz/4.2 = 59.5 MIPS |

The parts refer to pipelined machine only

(c) What is the instruction latency? (PAGE 522) with no dependancies, is the amount of time it is in the pipeline: 6

(d) What is the pipeline clock? slowest stage = 1/4ns = 250 MHz

(e) What is the pipelined speed up? page 440, non-pipelined (single)/pipelined = 11 ns/4ns = 2.75

(f) Without forwarding, fill in the hazard type (page 441-8) and condition (page 479), for the following instruction sequence:

| Instruction | Hazard type |
| --- | --- |
| add $1, $2, $3 | none |
| sub $4, $2, $1 | data hazard |
| lw $5, data($6) |  |
| beq $1, $5, loop2 | two data hazards |

(g) Without forwarding, draw the pipeline sequence (like figure 6.4) for part f

(h) .Show all timing, including stalls.

| Time | 1 2 3 4 5 6 7 8 9 10 11 12 |
| --- | --- |
| add $1, $2, $3 | IF ID EX EX2 MEM WB |
| sub $4, $2, $1 | IF ID ID ID ID EX EX2 MEM WB |
| lw $5, data($6) | IF ID EX EX2 MEM WB |
| beq $1, $5, loop2 | IF ID ID ID ID EX EX2 MEM WB |

(i) Suppose the hazard detection unit does not work on the processor due to a bug in the design.  Using the nop instruction, rewrite the code of part f which will give correct results (like page 478).

| Time | 1 2 3 4 5 6 7 8 9 10 11 12 … |
| --- | --- |
| add $1, $2, $3 | IF ID EX EX2 MEM WB |
| nop | IF ID EX EX2 MEM WB |
| nop | IF ID EX EX2 MEM WB |
| nop | IF ID EX EX2 MEM WB |
| sub $4, $2, $1 | IF ID EX EX2 MEM WB |
| lw $5, data($6) | IF ID EX EX2 MEM WB |
| nop | IF ID EX EX2 MEM WB |
| nop | IF ID EX EX2 MEM WB |
| nop | IF ID EX EX2 MEM |
| beq $1, $5, loop2 | IF ID EX EX2 |

(j) Using forwarding, draw graphical pipeline sequence (like figure 6.8) of part f. Show all shading.

(k) Show all timing including stalls.

(l) Using forwarding, reorder the code to minimize the stalling of part f.

lw	$5, data($6)			move non-dependant load as early as possible!

add	$1, $2, $3

sub	$4, $2, $1

beq	$1, $5, loop2

(m) Given any 2 instruction combinations, what is the worst instruction sequence. (i.e. assume no forwarding or prediction). Draw the pipeline chart (like Figure 6.4). Determine the MIPS.

- In general data dependencies create the worst sequence. Here are two observations:

- The arithmetic or R-format instructions can be forwarded early because the alu result is early in the pipeline stage (i.e. EX or EX2 stages).

- The data result of the load is the worse than the R-format because the result can only come after the memory has been read (i.e. MEM stage).

| Time | 1 2 3 4 5 6 7 8 9 10 11 12 |
| --- | --- |
| lw $1, 10($2) | IF ID EX EX2 MEM WB |
| lw $2, 20($1) | IF ID ID ID ID EX EX2 MEM WB |

Assume %100 percent dependancy:   then instruction time is 1 + 3 stalls = 4 clocks

Then MIPS = clock speed/CPI = 250 MHz/4 = 62.5 MIPs

(n) Given any 2 instruction combinations, what is the worst instruction sequence using forwarding. Draw the pipeline chart (like Figure 6.4). Determine the MIPS.

| Time | 1 2 3 4 5 6 7 8 9 10 11 12 |
| --- | --- |
| lw $1, 10($2) | IF ID EX EX2 MEM WB |
| lw $2, 20($1) | IF ID ID ID EX EX2 MEM WB |

Assume %100 percent dependancy:   then instruction time is 1 + 2 stalls = 3 clocks

Then MIPS = clock speed/CPI = 250 MHz/3 = 83.3 MIPs

(o) Using all the best possible instruction flow (i.e. no hazards, assume no forwarding or prediction) of the 5 instructions.  Determine the MIPS.

That would be any set of instructions that are not data dependant (0% dependancy).

Then the instruction time = 1 clock

MIPS = Clock speed/CPI = 250 Mhz / 1 = 250 MIPs

(p) Show the worst possible instruction pipeline sequence of using only the 5 instructions. (i.e. assume no forwarding or prediction). Draw the pipeline chart (Figure 6.4). Determine the MIPS.

That would be that case of 100% dependency like part m. Same solution for MIPs as part m.

lw 	$1, 10($2)

lw	$1, 10($1)

lw	$1, 10($1)

lw	$1, 10($1)

lw	$1, 10($1)