| EECS 322 | # Test 4 | ## Wednesday April 12, 2000 |
| --- | --- | --- |

| Name: |  |  |
| --- | --- | --- |

Problem 1 (30%).  Show each step of the pipeline machine (page 470 and 469) for the following instruction sequence:

Assume $1=9, $2=8; Mem[12]=16. Treat the “nop” instruction as “add $0,$0,$0”.

sw	$1, 8($2)

lw	$1, 4($2)

nop

nop

| Clock Both solutions ok X = do not care or 36 | <IF/ID> <PC, IR> | <ID/EX> <WB,M,EX,PC,A,B,S,Rt,Rd> | <EX/MEM> <WB,M,PC,Z, ALU, B, R> | <MEM/WB> <WB,MDR,ALU,R> |
| --- | --- | --- | --- | --- |
| 0 | <0,?> | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 1 | <4,”sw $1,8($2)”> observe that sw $rt=$1,8($rs=$2) | <?,?,?,?,?,?,?,?,?> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 2 | <8,”lw $1,4($2)”> observe that lw $rt=$1,4($rs=$2) | <0X, 001, X001, 4, 8, 9, 8, $1,X> observe that <wb=1X, m=001, ex=X001, pc=4, $rs$28A, $rt$19B, S=4, $rt=$1, $rd=X> | <?,?,?,?,?,?,?> | <?,?,?,?,?> |
| 3 | <12, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <11, 010, 0001, 8, 8, 9, 4, $1,X> observe that <wb=11 ,m=010, ex=0001, pc=8, $rs$28A, $rt$19B, S=4, $rt=$1, $rd=X> | <0X, 001, X or 36, X or 0, 16, 9, $1> observe that <wb=1X, m=001, pc=pc+(S<<2)=4+(8<<2)=36, Z=0, ALU=S+$2=8+8=16, 9, $1> ### Store to memory Mem[ALUOut]  B #### Mem[16]  9 | <?,?,?,?,?> |
| 4 | <16, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 12, 0, 0, 0, $0, $0> observe that <wb=10, m=000, ex=1100, pc=12, $rs=$0=0=A, $rt=$0=0=B, S=0, $rt=$0, $rd=$0> | <11, 010, X or 24, X or 0, 12, 9, $1> observe that <wb=11, m=010, pc=pc+(S<<2)=8+(4<<2)=24, Z=0, ALU=S+$2=4+8=12, 9, $1> ### Load from memory Mem[12]16 | <0X, X, X or 16, X or $1> Nothing happens here! observe that <wb=0X, mdr=X, ALU=16, R=$1> |
| 5 | <20, “nop”> observe that add $rd=0,$rs=0,$rt=0 | <10, 000, 1100, 16, 0, 0, 0, $0, $0> observe that <wb=10, m=000, ex=1100, pc=16, $rs=$0=0=A, $rt=$0=0=B, S=0, $rt=$0, $rd=$0> | <10,000,X,0,0,0,$0> observe that <wb=10, m=000, pc=X, Z=0, ALU=A+B=0+0=0, B=0, R=$0> | <11, 16, 12, $1> =>Reg[$1]16 finally register 1 will contain the load memory word of 4 + $12 observe that <wb=11, mdr=Mem[ALU] =Mem[12]=16, ALU=12, R=$1> |

Problem 2 (50%).  Assume a simple 5 stage pipeline with the following execution times

| 1 | IF | Instruction fetch | 3 ns |
| --- | --- | --- | --- |
| 2 | ID | Register Read | 1 ns |
| 3 | EX | ALU | 3 ns; Branch decision made here |
| 4 | MEM | Data Access | Memory Write time=5 ns, Read time=4 ns |
| 5 | WB | Register Write | 2 ns |

This computer has the following instructions:

| Instruction | Operation |
| --- | --- |
| add $rd, $rs, $rt | $rd = $rs + $rt |
| lw $rt, addr16($rs) | $rt = Mem[addr16+$rs] |
| sw $rt, addr16($rs) | Mem[addr16+$rs]=$rt |
| beq $rs, $rt, disp16 | pc = pc+2+($rs-$rt=0?disp16:0) |

2a (10%) Fill in the following tables

| Instruction | IM | ID | EX | MEM | WB | Total Time | Multi- Cycles | Instruction Mix |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add | 3 ns | 1 ns | 3ns |  | 2 ns | 9 ns | 4 | 50% |
| lw | 3 ns | 1 ns | 3 ns @+$rs | 4 ns | 2 ns | 13 ns | 5 | 20% |
| sw | 3 ns | 1 ns | 3 ns @+$rs | 5 ns |  | 12 ns | 4 | 10% |
| beq | 3 ns | 1 ns | 3 ns |  |  | 7 ns | 3 | 20% |

2b (2%) Fill the following table and show work.

| Instruction | Clock frequency | CPI | MIPS |
| --- | --- | --- | --- |
| Single-cycle CPU | 1/13 ns = 76.9 MHz | =1 | 76.9 MHz/1 = 76.9 MIPS |
| multi-cycle CPU | 1 / 5 ns = 200 MHz | 4*50%+5*20%+4*10%+3*20% = 4 | 200 MHz/4 = 50 MIPS |

The following parts refer to the pipelined machine only

For the following code: Assume no forwarding.

2c (10%) Draw lines showing all the data dependencies in column 1.

and show the pipeline sequence (IF, ID, EX, M, WB) for the following code

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw $1, 4($2) | IF | ID | EX | M | WB |  |  |  |  |  |  |  |
| lw $3, 8($1) |  | IF | ID | ID | ID | EX | M | WB |  |  |  |  |
| sub $4, $1, $2 |  |  | IF | IF | IF | ID | EX | M | WB |  |  |  |
| add $5, $4, $2 |  |  |  |  |  | IF | ID | ID | ID | EX | M | WB |

2d (2%) How many clock cycles will a load take if there is a dependency?

Latency instruction time = 5+2 stalls = 7   or   Pipeline or Throughput clocks = 1+ 2 stalls = 3

2e (2%) How many clock cycles will a load take if there is no dependency?

Latency instruction time = 5+0 stalls = 5   or   Pipeline or Throughput clocks = 1 + 0 stalls = 1

Solution #1

Solution #2

2f (10%) Using forwarding, show the pipeline sequence (IF, ID, EX, M, WB)

and draw lines showing the forwarding.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lw $1, 4($2) | IF | ID | EX | M | WB |  |  |  |  |  |  |  |
| lw $3, 8($1) |  | IF | ID | ID | EX | M | WB |  |  |  |  |  |
| sub $4, $1, $2 |  |  | IF | IF | ID | EX | M | WB |  |  |  |  |
| add $5, $4, $2 |  |  |  |  | IF | ID | EX | M | WB |  |  |  |

2g (10%) For the following code: Assume no forwarding and no branch prediction.

Draw lines showing all the data dependencies and show the pipeline sequence (IF, ID, EX, M, WB)

and draw lines showing the forwarding. Note: Branch decision is made in the EX stage.

| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| beq $1, $2, loop | IF | ID | EX | M | WB |  |  |  |  |  |  |  |
| sub $3, $4, $5 |  |  |  | IF | ID | EX | M | WB |  |  |  |  |
| add $4, $4, $5 |  |  |  |  | IF | ID | EX | M | WB |  |  |  |

No branch prediction:

On IF stage: the Branch IF/ID stops fetching next instruction.

2h (2%) What is the pipeline clock?  = 1/5ns = 200 MHz (slowest pipeline stage)

2i  (2%) What is the pipelined speed up from a single-cycle processor?

speedup = single-cycle clock / pipelined clock = 13 ns / 5 ns = 2.6

Problem 3 (20%). Show all calculations for the following questions.

Assume a load word takes 2 cycle if no dependancy and if dependant then 5 clocks

Assume there is a 20% data dependency.

Assume a branch takes 2 cycle if true prediction and if false prediction then 5 clocks.

Assume that 25% of the branches are mispredicted.

3a (5%) What is the average load instruction time in clocks?

= 80%  (2 clocks) + 20%  (5 clocks) = 2.6 clocks

3b (5% What is the average branch instruction time in clocks?

= 75%  (2 clocks) + 25%  (5 clocks) = 2.75 clocks

3c (10%) Now fill in the table and show calculations.

| Instruction | Pipeline Cycles | Instruction Mix |
| --- | --- | --- |
| add | 1 | 50% |
| lw | 2.6 | 20% |
| sw | 1 | 10% |
| beq | 2.75 | 20% |
| Clock | 300 Mhz |  |
| CPI | = 1 50% + 2.6  20% + 1  10% + 2.75  20% = 1.67 |  |
| MIPs | 300 Mhz/ 1.67 = 179.6 MIPS |  |