| EECS 322 | # Test 5 sample | ## Monday May 1, 2000 |
| --- | --- | --- |

| Name: |  |  |
| --- | --- | --- |

Problem 1 (20%). Given the following direct-mapped cache architecture:

All instructions use byte addresses. The address bus is 8 bits. A word size is 16 bits.

Total data cache size 4 words.

1a. (2%) How many bits is the index?

= log2 cache size in words = log2 4 = 2

1b. (2%) How many bits is the byte offset?

= log2 number of bytes per word = log2 word size/8 = log2 16/8 = log2 2 = 1

1c. (2%) How many bits is the tag size?

= address size – index size – byte offset size = 8 – 2 – 1 = 5

1d. (14%) For the following instruction sequence, fill in the access bits to the data cache

| tag | index | byte offset | instruction |
| --- | --- | --- | --- |
| 00100 | 00 | 0 | lw $1, 32($0) |
| 00100 | 01 | 0 | lw $2, 34($0) |
|  |  |  | add $3, $1, $2 |
| 00101 Does not access data memory. Therefore does not access the data cache! | 10 | 0 | sw $3, 44($0) |
| 00001 | 10 | 0 | lw $4, 12($0) |
| 00101 | 00 | 1 | lbu $5, 41($0) |
|  |  |  | beq $1, $2, 34($0) |

Problem 2 (10%). Given the following 2-way set associative cache architecture:

All instructions use byte addresses. The address bus is 8 bits. A word size is 16 bits.

Total data cache size 4 words.

2a. (2%) How many bits is the index?

= log2 (cache size in words / N-way) = log2 (4/2) = log2 2= 1

2b. (2%) How many bits is the byte offset?

= log2 number of bytes per word = log2 word size/8 = log2 16/8 = log2 2 = 1

2c. (2%) How many bits is the tag size?

= address size – index size – byte offset size = 8 – 1 – 1 = 6

2d. (4%) For the following instruction sequence, fill in the access bits to the data cache

| tag | index | byte offset | instruction |
| --- | --- | --- | --- |
| 001000 | 0 | 0 | lw $1, 32($0) |
| 001000 | 0 | 1 | lbu $2, 33($0) |

Problem 3. (15%) For the following instruction sequence fill in the direct-mapped cache

The word size is 16 bits.

Memory[0]=0x1066; Memory[2]=0x1453; Memory[16]=$3=0x1776; Memory[20]=0x1914;

3a. (5%) Fill in the miss cache column.

| tag | index | byte offset | instruction | Valid or Tag Cache Miss? |
| --- | --- | --- | --- | --- |
| 000 | 00 | 0 | lw $1, 0($0) | yes - valid |
| 000 | 01 | 0 | lw $2, 2($0) | yes – valid |
| A store never creates a cache miss! Cache misses are a property of reads 010 | 00 | 0 | sw $3, 16($0) |  |
| Load byte must first read the word from memory (20) and then separates out the byte. 010 | 10 | 1 | lbu $5, 21($0) | yes |
| 010 | 10 | 0 | lw $6, 20($0) |  |

3b. (10%) The final state of the direct mapped data cache is:

| index | valid | tag | data |
| --- | --- | --- | --- |
| 00 | NY | 000010 | 0x10660x1776 |
| 01 | NY | 000 | 0x1453 |
| 10 | NY | 010 | 0x1914 |
| 11 | N |  |  |

Problem 4. (15%) For the following instruction sequence fill in the 2-way set associative LRU cache

The word size is 16 bits.

Memory[0]=0x1066; Memory[2]=0x1453; Memory[16]=$3=0x1776; Memory[20]=0x1914;

4a. (5%) Fill in the miss cache column.

| tag | index | byte offset | instruction | Valid or tag Cache Miss? |
| --- | --- | --- | --- | --- |
| 000 | 0 | 0 | lw $1, 0($0) | yes |
| 100 | 0 | 0 | sw $3, 16($0) |  |
| 000 | 0 | 1 | lbu $5, 1($0) |  |
| 101 | 0 | 0 | lw $6, 20($0) | yes – flush oldest reference |
| 000 | 1 | 0 | lw $1, 2($0) | yes |

4b. (10%) The final state of the 2-way set associative LRU data cache is:

| index | valid | tag | data |
| --- | --- | --- | --- |
| 0 | NY | 000 | 0x1066 (accessed by lw and lbu) |
|  | NY | 100101 | 0x17760x1914 |
| 1 | NY | 000 | 0x1453 |
|  | N |  |  |

Problem 5. (10%) Given a 1-word cache entry block size and one word wide memory bus organization (figure 7.13a, page 561), and the following access times:

1 clock cycle to send the address,

8 clock cycles to read access DRAM, 16 clock cycles to write to DRAM

1 clock cycle to to send a word

5a. (5%) What is the miss penalty for a write-through direct mapped cache?

The miss penalty only include data reads not writes, since write-through is handled on the store instruction.

Miss penalty = (1 send address) + (8 clocks to read) + (1 send word) = 10 clock cycles

5b. (5%) What is the miss penalty for a write-back direct mapped cache?

This miss penalty includes only data reads BUT on write back the cache entry may contain a block not previously written out before a new entry can be read in (page 554).

Miss penalty = (1 send address) +(1 send word ) + 1(16 clocks to write)

+ (1 send address) + (8 clocks to read) + (1 clock to send word) = 28 clocks

Problem 6 (20%). Given the following virtual memory architecture:

All instructions use byte addresses. The virtual address bus is 16 bits. A word is 16 bits.

Total page size 8 bytes. The real memory address bus is 16 bits.

6a. (2%) How many bits is the page offset?

= log2 number of bytes per page = log2 8 = log2  23 = 3

6b. (2%) How many bits is the physical page number size?

= virtual address bit size – page offset bit size = 16 – 3 = 13

6c. (2%) How many page table entries?

= virtual address size/page size = 216 / 23 = 213

6d. (2%) How large is the page table?

= #entries  (bytes per entry to hold the real memory address)

= 213  21  = 214 bytes = 16,384 bytes (i.e. a quarter the size of memory!)

6e. (12%) For the following instruction sequence, fill in the data access bits to the page table

| virtual page number | page offset | instruction |
| --- | --- | --- |
| 0000 0000 00100 | 000 | lw $1, 32($0) |
| 0000 0000 00100 | 010 | lw $2, 34($0) |
| 0000 0000 00101 | 001 | lbu $5, 41($0) |

Problem 7. (10%) Assume 2K real memory, LRU, a page size of 1K and no pages loaded.

Fill in the page fault columns

| instruction | Page fault? | Flush which page? | Write flushed page to disk? | Load what new page |
| --- | --- | --- | --- | --- |
| lw $1, 32($0) | yes |  |  | 0..1023 |
| lw $2, 34($0) | no |  |  |  |
| lbu $5, 41($0) | no |  |  |  |
| sw $6, 1100($0) | yes |  |  | 1024..2047 |
| lw $7, 2000($0) | no |  |  |  |
| lw $1, 4100($0) | yes | 0..1023 (oldest) | No – only lw’s | 4096..5119 |
| lw $2, 32($0) | yes | 1024..2047 (oldest) | Yes – had a sw | 0..1023 |